# Slocum Daily Monitoring

## STAGE 1: Situation and Location Awareness

- Slocum Glider Mission Control Page (Situation Awareness)

  - Check for any updated notes and read them

  - Note last call in time and hours ago

  - Glance at battery and vacuum, any sensors there out of norm (sensors may be colored if out of norm)

- Check Google Earth (Location awareness)

  - Proper Overlays

  - Glider in Dangerous Area (shipping lane, high traffic, reef, wrecks, etc)

  - Speed Check, distance made good towards waypoint
ex: below, glider not making good distance towards waypoint, in this case .2 km / hr!

## Stage 2: Monitoring

- Slocum Glider Mission Control Page – data anaylsis

  - Data issues

    - Early altimeter returns

    - Vehicle Lag / missing data

  - CTD data sanity check, uniform density gradient

  - Take note of strange temperature anomalies

Compare to density plot

  - Check for thermal lag

  - Typical Backscatter (returns near bottom and in coastal / mixed environments) and not too noisy

  - No time lagged oxygen data and check for hypoxia zones

- Diagnostic Plots

  - Depth plot

    - Confirm good ballast

    - Confirm pressure sensors agree

    - Confirm not impacting bottom

    - Confirm no early altimeter hits

  - Engineering Plot

    - Confirm on pitch angles

    - Confirm roll is steady and same throughout mission

    - Check heading performance (correlate with GE speed and progress estimate)

  - Battery Diagnostic (primarily Aklaline)

    - Check for strange drops

    - Predict days remaining (distance we can travel = days remaining * speed made good)

  - Power / Energy Diagnostic (primary Lithium but relevant to Alkaline)