# Slocum Recovery – boat

- Contact shore support; inform when leaving dock and obtain lat/long of glider.

- Lat/long updates can be obtained via cell text, iridium text, or phone calls.

- Obtain Freewave communication w/ glider once vessel is in range.

- Obtain lat/long by typing command “where”.

- Recover glider (various methods).  Cart can be used, tail may be handled/roped, etc.  Exercise care with optical sensors and wings, as these can be easily damaged.

- Inform shore support that glider is recovered – “The bear is in the igloo.”

- Perform CTD comparison cast; note instrument type.  (Currently YSI Castaway or SBE 19).  Note lat/long of CTD cast.

- When instructed, remove green plug and insert red plug.  Secure glider for transport.